Changing U2 on main PCB of SVT-7 Pro

- Locate screws on chassis top

- Remove screws from chassis top

- Remove chassis top

- Disconnect cable at J1 of main board

- Cable disconnected at J1

- Locate blue and brown wire to Con3 and Con4

- Using needle nose pliers disconnect wires at Con3 and Con 4

- Locate and Disconnect wires from Con1 and Con2

- Locate and disconnect wires at Con5 and Con6

- Disconnect fan cable at J22

- Locate ties at front panel shield

- Cut these two cable ties

- Remove cut cable ties

- Cable at J13 on main passes through front shield.  Remove plastic grommet.

- Take grommet off cable

16.  Slide cable through hole.

17.Cable to J13 is now free from obstruction

18. Locate screws holding PCB to chassis.

19. Remove screws holding PCB to chassis

20. Remove Main PCB assembly by lifting front part of PCB

21. Main assembly removed by lifting and pulling forward to allow power wires to clear rear portion.

22. Locate U2

23. Locating U2 pins on bottom of PCB

24. Unsolder and remove U2.  Replace U2 with new improved part #....

25.  Reassemble.